Below is a TX MATLAB model aligned to the MathWorks UCIe 2.0 Transmitter/Receiver IBIS-AMI Models example. The official example targets single-ended data links, 32 GT/s, 31.25 ps symbol time, 16 samples/symbol, Standard package, TX swing = 0.625 V, TX rise time = 12.5 ps, TX impedance/capacitance = 30 Ω / 0.125 pF, and a channel with about 2 dB loss at 16 GHz. It starts from SerDes Designer and later supports IBIS-AMI export with selectable TX FFE settings.

TX behavioral model assumptions

This script keeps the same top-level assumptions as the MathWorks flow, but exposes them as plain MATLAB so you can modify the TX architecture directly for system studies. It models:

- PRBS/NRZ stimulus

- TX FFE with main + post-cursor tap

- finite TX output rise time

- TX output RC

- channel loss approximation

- RX-side loading, only as the far-end environment

- eye and BER estimation

The MathWorks uses selectable TX equalization settings and custom AMI parameters for TX configuration in the exported model flow.

PRBS/NRZ stimulus

The transmitter stimulus in the UCIe system-level model is generated using a Pseudo-Random Binary Sequence (PRBS), which closely emulates real data traffic patterns encountered in high-speed chiplet interconnects. PRBS sequences provide a statistically rich input that exercises the channel across a wide range of transitions, making them ideal for evaluating inter-symbol interference (ISI), jitter sensitivity, and equalization performance. In this model, the PRBS data is mapped into NRZ (Non-Return-to-Zero) signaling, where binary ‘0’ and ‘1’ are represented as two distinct voltage levels (typically ±1 normalized, later scaled to the TX swing).

The NRZ representation is widely adopted in UCIe due to its simplicity and compatibility with high-speed CMOS implementations. The generated bit sequence is then upsampled based on samples-per-symbol (e.g., 16 samples/UI) to create a continuous-time waveform suitable for analog processing stages such as FFE and driver modeling. This step ensures that waveform transitions and timing characteristics are accurately captured. The PRBS/NRZ stimulus forms the foundation for evaluating the entire TX chain, as it determines the spectral content, transition density, and stress conditions applied to the transmitter and channel.

TX FFE with main + post-cursor tap

The Feed-Forward Equalizer (FFE) is a critical component in the UCIe transmitter, designed to mitigate channel-induced ISI by pre-conditioning the transmitted waveform. In this model, a simplified two-tap FFE structure is used, consisting of a main tap (current symbol weight) and a post-cursor tap (previous symbol weight). The mathematical representation is:

y[n]=c0⋅x[n]+c1⋅x[n-1]

where c0is the main tap and c1is the post-cursor coefficient. The post-cursor tap introduces controlled de-emphasis to cancel ISI caused by channel memory effects.

In UCIe systems, multiple predefined FFE settings are typically available and selectable through configuration registers or AMI parameters. These presets allow optimization for different channel conditions (short, medium, long reach). The MATLAB model replicates this behavior through a configurable preset table, enabling rapid exploration of equalization trade-offs. Increasing post-cursor magnitude improves ISI cancellation but may reduce signal amplitude and increase power consumption. Thus, FFE design directly impacts eye opening, BER, and energy efficiency, making it a key tuning parameter in TX system optimization.

Finite TX Output Rise Time

The transmitter output does not exhibit ideal instantaneous transitions; instead, it is limited by a finite rise and fall time, typically defined by the bandwidth of the driver circuitry. In the UCIe reference model, the TX rise time is approximately 12.5 ps, which corresponds to a specific bandwidth limitation. This effect is modeled using a first-order low-pass filter, where the −3 dB bandwidth is approximated as:

f3dB≈0.35tr

Finite rise time introduces bandwidth limitations that attenuate high-frequency components of the signal, leading to increased ISI and reduced eye opening. This is particularly critical at 32 GT/s, where signal transitions occur within tens of picoseconds. The modeling of rise time ensures realistic waveform shaping and allows designers to evaluate the trade-off between signal integrity and power consumption, since faster edges typically require higher current drive and result in increased EMI.

In system-level simulations, incorporating rise time effects is essential for accurately predicting channel interaction and equalization effectiveness. It also provides insight into how driver design constraints influence overall link performance.

TX output RC

The transmitter output stage is modeled as a combination of output resistance (Rout) and parasitic capacitance (Cout), forming an RC network that further shapes the transmitted waveform. In the UCIe reference configuration, typical values are 30 Ω and 0.125 pF, representing the effective impedance and capacitance of the driver and pad interface.

This RC network introduces an additional low-pass filtering effect, which impacts signal edge rates and amplitude. The time constant is given by:

τ=Rout⋅Cout

and determines how quickly the output can respond to changes in the input signal. A larger RC time constant results in slower transitions and increased ISI, while a smaller value improves bandwidth but may lead to higher power consumption and signal ringing.

Modeling the TX output RC is important for capturing realistic driver behavior, including impedance matching and interaction with the channel. It also enables analysis of how packaging parasitics and layout choices influence signal integrity. In system-level studies, this block helps bridge the gap between ideal digital output and actual analog waveform characteristics seen at the transmitter pad.

channel loss approximation

The channel in a UCIe system consists of package interconnects, micro-bumps, and short interposer traces, which collectively introduce frequency-dependent attenuation. In the MATLAB model, the channel is approximated using a single-pole low-pass filter, calibrated to match a target insertion loss (e.g., 2 dB at 16 GHz, corresponding to Nyquist frequency for 32 GT/s NRZ).

This simplified model captures the essential behavior of the channel:

H(f)=11+(f/fp)2

where fpis chosen to match the desired loss at Nyquist. While this approach does not include detailed effects such as reflections or impedance discontinuities, it provides a computationally efficient way to evaluate system-level performance.

Channel loss directly affects high-frequency components, leading to eye closure and increased ISI. Accurate modeling is essential for determining the required FFE strength and assessing link margin. For more detailed analysis, this block can later be replaced with S-parameter-based models, but the low-order approximation is sufficient for early architecture exploration and parameter sweeps.

RX-side loading, only as the far-end environment

Although the focus is on the transmitter, the model includes a simplified representation of the receiver input as a far-end load, typically modeled as 50 Ω resistance with 0.125 pF capacitance. This loading condition reflects the electrical termination seen by the transmitted signal and plays a crucial role in determining signal amplitude and bandwidth.

The RX load interacts with the channel and TX output impedance, forming a complete signal path that influences waveform integrity. The capacitive component introduces additional filtering, while the resistive component ensures proper impedance matching to minimize reflections.

Including RX-side loading in the TX model allows for a more realistic evaluation of signal behavior at the receiver input, even without modeling the full RX equalization chain. It ensures that the transmitted waveform reflects actual operating conditions, enabling accurate estimation of eye opening, voltage swing, and BER. This approach is particularly useful in early-stage design when TX optimization is performed independently of detailed RX modeling.

eye and BER estimation

The final stage of the model evaluates link performance using eye diagrams and Bit Error Rate (BER) estimation. The eye diagram is generated by overlaying multiple unit intervals of the received waveform, providing a visual representation of signal integrity. Key metrics include eye height (voltage margin) and eye width (timing margin), which directly correlate with link reliability.

BER is computed by sampling the waveform at the optimal decision point and comparing the detected bits with the transmitted sequence. Additionally, a Q-factor-based estimation is used:

BER≈12⋅erfcQ2

where Qis derived from the separation between logic levels relative to noise variance.

This dual approach (measured BER + analytical estimate) provides both accuracy and insight into noise-limited performance. Eye and BER analysis enable designers to quantify the effectiveness of FFE, channel conditions, and noise levels, making them essential metrics for system validation and optimization. These outputs are directly aligned with IBIS-AMI simulation goals and are critical for ensuring compliance with UCIe link requirements.

MATLAB Model

%% UCIe 2.0 TX System-Level Behavioral Model

% Reference-aligned to MathWorks UCIe 2.0 IBIS-AMI example

% TX-only model with channel and far-end loading for eye/BER evaluation

%

% Main aligned assumptions from MathWorks example:

%   Data rate          = 32 GT/s

%   UI                 = 31.25 ps

%   Samples/UI         = 16

%   Data signaling     = Single-ended NRZ

%   TX swing           = 0.625 V

%   TX rise time       = 12.5 ps

%   TX Rout/Cout       = 30 ohm / 0.125 pF

%   Channel loss       = 2 dB @ 16 GHz

%   RX load            = 50 ohm / 0.125 pF

%

% This is a standalone architecture-level model, not an IBIS-AMI export script.

clc;

clear;

close all;

%% =========================================================

% 1) Top-level configuration

% ==========================================================

cfg.dataRate      = 32e9;                  % 32 GT/s

cfg.UI            = 1/cfg.dataRate;        % 31.25 ps

cfg.samplesPerUI  = 16;                    % as in MathWorks example

cfg.fs            = cfg.dataRate * cfg.samplesPerUI;

cfg.targetBER     = 1e-15;                 % target used in example flow

cfg.nBits         = 2^15;                  % simulation length

cfg.modulation    = 'NRZ';

% Time axis for waveform-domain processing

cfg.nSamples      = cfg.nBits * cfg.samplesPerUI;

cfg.t             = (0:cfg.nSamples-1) / cfg.fs;

fprintf('UCIe TX model\n');

fprintf('Data rate      : %.2f GT/s\n', cfg.dataRate/1e9);

fprintf('UI             : %.2f ps\n', cfg.UI*1e12);

fprintf('Samples/UI     : %d\n', cfg.samplesPerUI);

fprintf('Sample rate    : %.2f GHz\n\n', cfg.fs/1e9);

%% =========================================================

% 2) TX electrical configuration

% ==========================================================

tx.vSwing         = 0.625;                 % V

tx.riseTime       = 12.5e-12;              % 12.5 ps

tx.Rout           = 30;                    % ohm

tx.Cout           = 0.125e-12;             % F

% UCIe-like FFE presets (main + post)

% The MathWorks example uses TX equalization settings selectable in the model flow.

% These presets are a practical behavioral representation.

tx.ffePresets = [

1.00   0.00;    % preset 0: no de-emphasis

0.90  -0.10;    % preset 1

0.85  -0.15;    % preset 2

0.75  -0.25     % preset 3

];

tx.presetSel      = 3;                     % choose row from table above

tx.mainTap        = tx.ffePresets(tx.presetSel,1);

tx.postTap        = tx.ffePresets(tx.presetSel,2);

%% =========================================================

% 3) Channel and far-end environment

% ==========================================================

ch.loss_dB_16GHz  = 2.0;                   % channel target from example

ch.fRef           = 16e9;                  % 16 GHz

% Simple one-pole approximation to match 2 dB loss at 16 GHz

A16 = 10^(-ch.loss_dB_16GHz/20);           % linear magnitude at 16 GHz

ch.fp = ch.fRef / sqrt((1/A16^2)-1);       % solve |H(jw)| = A16 for 1-pole LPF

rxLoad.Rin        = 50;                    % ohm

rxLoad.Cin        = 0.125e-12;             % F

%% =========================================================

% 4) Noise and jitter knobs

% ==========================================================

sim.noiseSigma    = 5e-3;                  % V RMS additive noise

sim.enableJitter  = false;                 % set true for sampling jitter test

sim.rjSigma       = 0.25e-12;              % 0.25 ps RMS sample jitter

%% =========================================================

% 5) Data generation

% ==========================================================

% Random NRZ data

bits = randi([0 1], 1, cfg.nBits);

sym  = 2*bits - 1;                         % map to {-1, +1}

% Upsampled ideal waveform

xIdeal = repelem(sym, cfg.samplesPerUI);

%% =========================================================

% 6) TX FFE

%   y[n] = c0*x[n] + c1*x[n-1]

% ==========================================================

ffeSym = filter([tx.mainTap tx.postTap], 1, sym);

ffeWfm = repelem(ffeSym, cfg.samplesPerUI);

% Normalize to TX swing

% Use +/- vSwing/2 around zero

txFFE = (tx.vSwing/2) * ffeWfm;

%% =========================================================

% 7) TX edge shaping

% ==========================================================

% Approximate finite TX rise time with a first-order low-pass.

% For a single-pole system, f3dB ≈ 0.35 / tr

tx.f3dB_edge = 0.35 / tx.riseTime;

w3dB         = 2*pi*tx.f3dB_edge;

% Continuous-time H(s) = w3dB / (s + w3dB)

[num_edge_z, den_edge_z] = bilinear(w3dB, [1 w3dB], cfg.fs);

txEdge = filter(num_edge_z, den_edge_z, txFFE);

%% =========================================================

% 8) TX output RC

% ==========================================================

tau_tx = tx.Rout * tx.Cout;

alpha_tx = exp(-1/(cfg.fs*tau_tx));

txOut = filter(1-alpha_tx, [1 -alpha_tx], txEdge);

%% =========================================================

% 9) Channel model

% ==========================================================

% One-pole LPF fit to 2 dB at 16 GHz

wCh = 2*pi*ch.fp;

[num_ch_z, den_ch_z] = bilinear(wCh, [1 wCh], cfg.fs);

chOut = filter(num_ch_z, den_ch_z, txOut);

%% =========================================================

% 10) Far-end load

% ==========================================================

tau_rx = rxLoad.Rin * rxLoad.Cin;

alpha_rx = exp(-1/(cfg.fs*tau_rx));

rxIn = filter(1-alpha_rx, [1 -alpha_rx], chOut);

%% =========================================================

% 11) Add noise

% ==========================================================

rxNoisy = rxIn + sim.noiseSigma * randn(size(rxIn));

%% =========================================================

% 12) Symbol sampling

% ==========================================================

samplePhase = round(cfg.samplesPerUI/2);   % center-of-UI sample

sampleIdx   = samplePhase:cfg.samplesPerUI:length(rxNoisy);

sampleIdx   = sampleIdx(1:cfg.nBits);

if sim.enableJitter

% Optional random sample movement in units of samples

jitterSec = sim.rjSigma * randn(size(sampleIdx));

jitterSmp = round(jitterSec * cfg.fs);

sampleIdx = sampleIdx + jitterSmp;

sampleIdx(sampleIdx < 1) = 1;

sampleIdx(sampleIdx > length(rxNoisy)) = length(rxNoisy);

end

rxSamples = rxNoisy(sampleIdx);

%% =========================================================

% 13) BER and eye metrics

% ==========================================================

decBits = rxSamples > 0;

ber_meas = mean(decBits ~= bits);

oneS  = rxSamples(bits == 1);

zeroS = rxSamples(bits == 0);

mu1   = mean(oneS);

mu0   = mean(zeroS);

sig1  = std(oneS);

sig0  = std(zeroS);

eyeHeight = mu1 - mu0;

qEst = eyeHeight / (sig1 + sig0);

ber_q = 0.5 * erfc(qEst / sqrt(2));

%% =========================================================

% 14) Frequency responses

% ==========================================================

Nfft = 4096;

f = linspace(0, cfg.fs/2, Nfft/2+1);

Hedge = freqz(num_edge_z, den_edge_z, Nfft, cfg.fs);

Hch   = freqz(num_ch_z,   den_ch_z,   Nfft, cfg.fs);

%% =========================================================

% 15) Results

% ==========================================================

fprintf('Selected FFE preset  : [%0.2f  %0.2f]\n', tx.mainTap, tx.postTap);

fprintf('TX swing             : %.3f V\n', tx.vSwing);

fprintf('TX edge f3dB         : %.2f GHz\n', tx.f3dB_edge/1e9);

fprintf('Channel pole         : %.2f GHz\n', ch.fp/1e9);

fprintf('Measured BER         : %e\n', ber_meas);

fprintf('Q-based BER estimate : %e\n', ber_q);

fprintf('Eye height           : %.4f V\n', eyeHeight);

%% =========================================================

% 16) Plots

% ==========================================================

% (a) Symbol-domain FFE output

figure;

plot(ffeSym(1:100), 'o-','LineWidth',1.0);

grid on;

xlabel('Symbol index');

ylabel('FFE output (normalized)');

title('TX FFE Symbol Output');

% (b) TX time waveform

figure;

plot(cfg.t(1:2000)*1e12, txOut(1:2000), 'LineWidth',1.2);

grid on;

xlabel('Time (ps)');

ylabel('Voltage (V)');

title('TX Output Waveform');

% (c) Channel output

figure;

plot(cfg.t(1:2000)*1e12, rxNoisy(1:2000), 'LineWidth',1.2);

grid on;

xlabel('Time (ps)');

ylabel('Voltage (V)');

title('Waveform After Channel + Far-End Load');

% (d) Frequency response

figure;

plot(f/1e9, 20*log10(abs(Hedge(1:length(f)))), 'LineWidth',1.2); hold on;

plot(f/1e9, 20*log10(abs(Hch(1:length(f)))),   'LineWidth',1.2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('TX Edge Shaping','Channel','Location','best');

title('TX/Channel Frequency Responses');

% (e) Eye diagram without requiring eyediagram()

nUI_eye  = 2;

Leye     = nUI_eye * cfg.samplesPerUI;

nTraces  = floor(length(rxNoisy)/cfg.samplesPerUI) - 2;

tEye     = (0:Leye-1) / cfg.samplesPerUI;

figure; hold on; grid on;

for k = 1:min(nTraces, 800)

idx0 = (k-1)*cfg.samplesPerUI + 1;

seg = rxNoisy(idx0:idx0+Leye-1);

plot(tEye, seg, 'b');

end

xlabel('UI');

ylabel('Voltage (V)');

title('Eye Diagram at Receiver Input');

xlim([0 nUI_eye]);

% (f) Sample histogram

figure;

histogram(zeroS, 60, 'Normalization','pdf'); hold on;

histogram(oneS,  60, 'Normalization','pdf');

grid on;

xlabel('Sample voltage (V)');

ylabel('PDF');

legend('Bit 0','Bit 1','Location','best');

title('Sampled Voltage Distributions');